https://www.mycompiler.io/new/octave

Problem 1: Simulation and Analysis of 16-QAM Modulation in 5G NR

Implement and analyze a 16-QAM modulation scheme for 5G NR systems. Evaluate the Bit Error Rate (BER) performance over an AWGN channel and plot BER versus SNR.

- Develop a MATLAB/Octave simulation of 16-QAM modulation and demodulation.

- Transmit data through an AWGN channel.

- Analyze BER performance as a function of SNR.

Program:

clc;

clear;

close all;

% Parameters

M = 64; % Modulation order (4, 16, 64)

k = log2(M); % Bits per symbol

num_bits = 1000; % Total number of bits

% Generate random bits

bits = randi([0 1], num_bits, 1);

% Ensure length of bits is a multiple of k

num_symbols = floor(length(bits) / k);

bits = bits(1:num_symbols * k);

% Reshape bits into k-bit groups (each row = 1 symbol)

bit_groups = reshape(bits, k, num_symbols).';

% Manual binary to decimal conversion (replace bi2de)

powers_of_two = 2.^(k-1:-1:0);

symbol_indices = bit_groups * powers_of_two.';  % decimal symbol indices

% Generate QAM constellation points (square QAM only)

m_sqrt = sqrt(M);

if mod(m_sqrt, 1) ~= 0

error('M must be a perfect square for square QAM');

end

% Create I and Q points

I = -(m_sqrt - 1):2:(m_sqrt - 1);

Q = -(m_sqrt - 1):2:(m_sqrt - 1);

% Create meshgrid and flatten it

[II, QQ] = meshgrid(I, Q);

constellation = II(:) + 1j * QQ(:);

% Normalize constellation power to 1

constellation = constellation / sqrt(mean(abs(constellation).^2));

% Modulate: map indices to constellation points

tx_symbols = constellation(symbol_indices + 1);

% Plot transmitted symbols (Constellation Diagram)

figure;

scatter(real(tx_symbols), imag(tx_symbols), 'filled');

xlabel('In-Phase');

ylabel('Quadrature');

title([num2str(M) '-QAM Constellation (Transmitted Symbols)']);

grid on;

% Add AWGN Noise (Optional)

snr_db = 20; % SNR in dB

snr_linear = 10^(snr_db/10);

noise_variance = 1 / (2 * snr_linear);

noise = sqrt(noise_variance) * (randn(size(tx_symbols)) + 1j * randn(size(tx_symbols)));

rx_symbols = tx_symbols + noise;

% Demodulate: Find nearest constellation point

distances = abs(rx_symbols - constellation.');

[~, demod_indices] = min(distances, [], 2);

% Manual decimal to binary conversion (replace de2bi)

demod_bits = zeros(num_symbols, k);

for i = 1:k

demod_bits(:, i) = bitget(demod_indices - 1, k - i + 1);

end

% Reshape to column vector

demod_bits = reshape(demod_bits.', [], 1);

% Compute BER (Bit Error Rate)

num_errors = sum(bits ~= demod_bits);

ber = num_errors / length(bits);

% Display results

disp(['Number of bit errors: ' num2str(num_errors)]);

disp(['Bit Error Rate (BER): ' num2str(ber)]);

% Plot received symbols (With noise)

figure;

scatter(real(rx_symbols), imag(rx_symbols), 'filled');

xlabel('In-Phase');

ylabel('Quadrature');

title([num2str(M) '-QAM Constellation (Received Symbols with Noise, SNR = ' num2str(snr_db) ' dB)']);

grid on;

Results:

M=4

Number of bit errors: 0

Bit Error Rate (BER): 0

M=16

Number of bit errors: 0

Bit Error Rate (BER): 0

M=64

Number of bit errors: 11

Bit Error Rate (BER): 0.011044

Problem 2: Adaptive Modulation and Coding (AMC) in 5G NR

Simulate an Adaptive Modulation and Coding (AMC) mechanism in 5G NR that dynamically selects modulation schemes (BPSK, QPSK, 16-QAM) based on varying SNR conditions.

- Implement SNR-based modulation adaptation.

- Evaluate BER and throughput performance for each modulation type.

- Demonstrate improved system efficiency through adaptive selection.

Program:

clc;

clear;

% Define SNR range

snr_range = 0:2:20;

% Thresholds for adaptive modulation

snr_threshold_qpsk = 5;

snr_threshold_16qam = 15;

% Initialize result storage

bits_per_symbol_all = zeros(size(snr_range));

% Loop over SNR range

for idx = 1:length(snr_range)

snr = snr_range(idx);

% Adaptive modulation selection

if snr < snr_threshold_qpsk

M = 2; % BPSK

scheme = 'BPSK';

elseif snr < snr_threshold_16qam

M = 4; % QPSK

scheme = 'QPSK';

else

M = 16; % 16-QAM

scheme = '16-QAM';

end

% Save results

bits_per_symbol = log2(M);

bits_per_symbol_all(idx) = bits_per_symbol;

% Display decision

disp(['SNR: ', num2str(snr), ' dB - Modulation: ', scheme, ' - Bits per symbol: ', num2str(bits_per_symbol)]);

end

% Plot bits per symbol vs SNR

figure;

plot(snr_range, bits_per_symbol_all, '-ob', 'LineWidth', 2, 'MarkerSize', 6);

grid on;

xlabel('SNR (dB)');

ylabel('Bits per Symbol');

title('Adaptive Modulation: Bits per Symbol vs SNR');

Result:

SNR: 0 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 2 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 4 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 6 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 8 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 10 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 12 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 14 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 16 dB - Modulation: 16-QAM - Bits per symbol: 4

SNR: 18 dB - Modulation: 16-QAM - Bits per symbol: 4

SNR: 20 dB - Modulation: 16-QAM - Bits per symbol: 4

Problem 3: 5G NR Channel Coding using Parity-Check Codes

Implement basic channel coding in 5G NR using parity-check codes. Demonstrate error detection capability over an AWGN channel.

- Encode data using parity-check coding.

- Simulate transmission over an AWGN channel.

- Implement error detection and evaluate BER improvement.

Program:

clc;

clear;

% Parameters

M = 4; % QPSK

num_bits = 10000;

SNR_dB = 0:5:30;

BER = zeros(size(SNR_dB));

% Generate random bits

bits = randi([0 1], num_bits, 1);

% Manual QPSK Modulation

symbols = [];

for i = 1:2:num_bits

b1 = bits(i);

b2 = bits(i+1);

re = 2*b1 - 1;

im = 2*b2 - 1;

symbols = [symbols; (re + 1i*im)/sqrt(2)];

end

for idx = 1:length(SNR_dB)

snr = SNR_dB(idx);

% Add AWGN

noise = (randn(size(symbols)) + 1i*randn(size(symbols))) * 10^(-snr/20);

rx = symbols + noise;

% Demodulation

rx_bits = [];

for k = 1:length(rx)

re = real(rx(k));

im = imag(rx(k));

rx_bits = [rx_bits; re > 0; im > 0];

end

% BER

errors = sum(bits ~= rx_bits);

BER(idx) = errors / num_bits;

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB, BER, 'bo-');

xlabel('SNR (dB)');

ylabel('BER');

title('QPSK over AWGN');

grid on;

Result:

Problem 4: Basic OFDM Transmitter and Receiver Simulation for 5G NR

Implement a basic OFDM-based transmitter and receiver for 5G NR. Perform BER analysis over an AWGN channel.

- Simulate OFDM modulation (IFFT) and demodulation (FFT).

- Transmit data through an AWGN channel.

- Analyze BER versus SNR.

Program :

clc;

clear;

% --------------------------

% Parameters

% --------------------------

N = 64;              % Number of subcarriers

CP_len = 16;         % Cyclic prefix length

num_symbols = 100;   % Number of OFDM symbols per SNR

M = 4;               % QPSK (2 bits per symbol)

SNR_dB_range = 0:2:20; % SNR range in dB for BER plot

num_bits_per_symbol = log2(M);

num_total_bits = num_symbols * N * num_bits_per_symbol;

BER = zeros(length(SNR_dB_range), 1); % Initialize BER array

% --------------------------

% Start BER vs. SNR Simulation

% --------------------------

for idx = 1:length(SNR_dB_range)

snr_db = SNR_dB_range(idx);

% --------------------------------------

% Transmitter: Generate random data bits

% --------------------------------------

bits = randi([0 1], num_total_bits, 1);

% --------------------------------------

% QPSK Modulation (manual)

% --------------------------------------

symbols = zeros(length(bits)/2, 1);

for i = 1:2:length(bits)

b1 = bits(i);

b2 = bits(i+1);

% QPSK mapping: Gray coding

re = 2 * b1 - 1;

im = 2 * b2 - 1;

symbols((i+1)/2) = (re + 1i * im) / sqrt(2);

end

% --------------------------------------

% OFDM Modulation

% --------------------------------------

ofdm_symbols = reshape(symbols, N, num_symbols);      % Reshape to N subcarriers

tx_signal = ifft(ofdm_symbols);                       % IFFT for OFDM modulation

tx_cp = [tx_signal(end - CP_len + 1:end, :); tx_signal]; % Add cyclic prefix

tx_serial = tx_cp(:);                                 % Serialize

% --------------------------------------

% Add AWGN Noise (manual)

% --------------------------------------

signal_power = mean(abs(tx_serial).^2);

snr_linear = 10^(snr_db / 10);

noise_power = signal_power / snr_linear;

noise = sqrt(noise_power / 2) * (randn(size(tx_serial)) + 1i * randn(size(tx_serial)));

rx_serial = tx_serial + noise;

% --------------------------------------

% OFDM Demodulation

% --------------------------------------

rx_cp = reshape(rx_serial, N + CP_len, num_symbols);

rx_signal = rx_cp(CP_len + 1:end, :);

rx_ofdm_symbols = fft(rx_signal);

rx_symbols = rx_ofdm_symbols(:);

% --------------------------------------

% QPSK Demodulation (manual)

% --------------------------------------

rx_bits = zeros(length(bits), 1);

for k = 1:length(rx_symbols)

re = real(rx_symbols(k));

im = imag(rx_symbols(k));

rx_bits(2*k - 1) = re > 0; % First bit (real part)

rx_bits(2*k)     = im > 0; % Second bit (imag part)

end

% --------------------------------------

% BER Calculation

% --------------------------------------

num_errors = sum(bits ~= rx_bits);

BER(idx) = num_errors / length(bits);

disp(['SNR = ' num2str(snr_db) ' dB, BER = ' num2str(BER(idx))]);

end

% --------------------------

% Plot BER vs. SNR

% --------------------------

figure;

semilogy(SNR_dB_range, BER, '-o', 'LineWidth', 2);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('BER vs. SNR for QPSK-OFDM (No External Functions)');

legend('OFDM-QPSK');

Result:

Problem 5: OFDM Transmission over Multipath Fading Channels with Cyclic Prefix in 5G NR

Simulate a 5G NR OFDM system with Cyclic Prefix (CP) and evaluate its performance over multipath fading channels.

- Implement Cyclic Prefix addition and removal.

- Model a multipath fading channel.

- Analyze BER performance under various SNRs and multipath conditions.

Program:

clc;

clear;

% Parameters

num_subcarriers = 64;

num_symbols = 50;  % Reduced for faster simulation

cp_len = 16;

SNR_dB_range = 0:5:30;

% Multipath Channel

h = [0.8; 0.5*exp(1j*pi/4); 0.3*exp(1j*pi/3)];

% BER Initialization

BER = zeros(1, length(SNR_dB_range));

% SNR loop

for snr_idx = 1:length(SNR_dB_range)

SNR_dB = SNR_dB_range(snr_idx);

errors = 0;

total_bits = 0;

% Generate bits

num_bits = num_subcarriers * num_symbols * 2; % 2 bits per QPSK symbol

bits = randi([0 1], num_bits, 1);

% QPSK modulation

symbols = zeros(num_subcarriers, num_symbols);

for sym_idx = 1:num_symbols

temp_symbols = zeros(num_subcarriers, 1);

for sc = 1:num_subcarriers

idx = (sym_idx - 1) * num_subcarriers * 2 + (sc - 1) * 2 + 1;

b1 = bits(idx);

b2 = bits(idx + 1);

temp_symbols(sc) = (2*b1 - 1 + 1j*(2*b2 - 1)) / sqrt(2);

end

symbols(:, sym_idx) = temp_symbols;

end

% OFDM Modulation + CP insertion

tx_signal = [];

for i = 1:num_symbols

ifft_data = ifft(symbols(:, i));

ofdm_symbol_cp = [ifft_data(end - cp_len + 1:end); ifft_data]; % Add cyclic prefix

tx_signal = [tx_signal; ofdm_symbol_cp];

end

% Channel convolution

channel_output = conv(tx_signal, h);

channel_output = channel_output(1:length(tx_signal));  % Trim extra samples

% Add AWGN

signal_power = mean(abs(channel_output).^2);

snr_linear = 10^(SNR_dB / 10);

noise_power = signal_power / snr_linear;

noise = sqrt(noise_power / 2) * (randn(size(channel_output)) + 1j * randn(size(channel_output)));

rx_signal = channel_output + noise;

% Receiver: OFDM Demod + CP removal + Equalization

rx_symbols = zeros(num_subcarriers, num_symbols);

index = 0;

for i = 1:num_symbols

start_idx = index + cp_len + 1;

end_idx = index + cp_len + num_subcarriers;

if end_idx > length(rx_signal)

disp('Reached end of rx signal');

break;

end

r = rx_signal(start_idx:end_idx);

index = index + num_subcarriers + cp_len;

rx_ofdm = fft(r);

% Channel frequency response

H = fft([h; zeros(num_subcarriers - length(h), 1)]);

% Zero Forcing Equalizer

eq_ofdm = rx_ofdm ./ H;

rx_symbols(:, i) = eq_ofdm;

end

% Demodulation

rx_bits = zeros(num_bits, 1);

symbol_counter = 0;

for i = 1:num_symbols

for k = 1:num_subcarriers

symbol_counter = symbol_counter + 1;

rx_sym = rx_symbols(k, i);

rx_bits(2*symbol_counter - 1) = real(rx_sym) > 0;

rx_bits(2*symbol_counter) = imag(rx_sym) > 0;

end

end

% BER calculation

bit_errors = sum(bits ~= rx_bits);

BER(snr_idx) = bit_errors / num_bits;

disp(['SNR = ', num2str(SNR_dB), ' dB, BER = ', num2str(BER(snr_idx))]);

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB_range, BER, 'ro-', 'LineWidth', 2);

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('OFDM BER vs SNR over Multipath Channel');

grid on;

legend('QPSK-OFDM');

Result:

SNR = 0 dB, BER = 0.20703

SNR = 5 dB, BER = 0.090781

SNR = 10 dB, BER = 0.026562

SNR = 15 dB, BER = 0.0034375

SNR = 20 dB, BER = 0

SNR = 25 dB, BER = 0

SNR = 30 dB, BER = 0

Problem 6: Exploring 5G NR Numerology (Subcarrier Spacing and CP Lengths)

Simulate different 5G NR numerologies by varying subcarrier spacing (SCS) and cyclic prefix lengths. Evaluate their impact on system performance.

- Implement multiple numerology configurations (e.g., 15 kHz, 30 kHz SCS).

- Analyze their effects on bandwidth, latency, and BER.

- Compare the trade-offs between different numerologies.

Program:

clc;

clear;

% Base parameters

f_ref = 15e3; % 15 kHz base subcarrier spacing

numerologies = 0:4; % µ = 0 to 4

fs = 30.72e6; % Sampling rate (for simplicity)

% Arrays to store results

SCS = zeros(size(numerologies));

CP_durations = zeros(size(numerologies));

for i = 1:length(numerologies)

mu = numerologies(i);

SCS(i) = f_ref * 2^mu; % Subcarrier spacing

Nfft = fs / SCS(i);

Ncp = Nfft * 0.07; % Assume CP is 7% of symbol length

CP_durations(i) = Ncp / fs * 1e6; % in microseconds

end

% Plot

figure;

subplot(2,1,1);

stem(numerologies, SCS/1e3, 'LineWidth', 2);

xlabel('Numerology Index (µ)');

ylabel('Subcarrier Spacing (kHz)');

title('5G NR Numerology: Subcarrier Spacing');

grid on;

subplot(2,1,2);

stem(numerologies, CP_durations, 'r', 'LineWidth', 2);

xlabel('Numerology Index (µ)');

ylabel('CP Duration (\mus)');

title('5G NR Numerology: CP Duration');

grid on;

Result:

Problem 7: Simulation of 2x2 MIMO Spatial Multiplexing in 5G NR

Implement and simulate a 2x2 MIMO system using spatial multiplexing and Zero Forcing detection for 5G NR. Evaluate BER performance over AWGN and Rayleigh fading channels.

- Develop the 2x2 MIMO channel model.

- Implement Zero Forcing (ZF) receiver.

- Analyze BER performance under varying SNRs.

Program:

clc;

clear;

num_bits = 1000;        % Reduced number of bits for faster execution

num_tx = 2;

num_rx = 2;

SNR_dB = 0:5:30;

BER = zeros(size(SNR_dB));

% Generate random bits

bits = randi([0 1], num_bits, 1);

% Manual QPSK Modulation

symbols = zeros(num_bits/2, 1);

for i = 1:2:num_bits

b1 = bits(i);

b2 = bits(i+1);

re = 2*b1 - 1;

im = 2*b2 - 1;

symbols((i+1)/2) = (re + 1i*im)/sqrt(2);

end

% Reshape into 2x streams

symbols_tx = reshape(symbols, num_tx, []);

num_symbols = size(symbols_tx, 2);

for idx = 1:length(SNR_dB)

snr = SNR_dB(idx);

errors = 0;

% Loop for all symbols

for n = 1:num_symbols

s = symbols_tx(:, n);

% Random channel matrix H

H = (randn(num_rx, num_tx) + 1i*randn(num_rx, num_tx)) / sqrt(2);

% Transmit signal through channel

y = H * s;

% Add noise

noise_power = 10^(-snr/10);

noise = sqrt(noise_power/2) * (randn(size(y)) + 1i*randn(size(y)));

y_noisy = y + noise;

% Zero Forcing Detection

s_est = pinv(H) * y_noisy;

% Demodulation

rx_bits = [];

for k = 1:length(s_est)

re = real(s_est(k));

im = imag(s_est(k));

rx_bits = [rx_bits; re > 0; im > 0];

end

% Original transmitted bits for comparison

tx_bits = [];

for k = 1:length(s)

re = real(s(k));

im = imag(s(k));

tx_bits = [tx_bits; re > 0; im > 0];

end

% Count bit errors

errors = errors + sum(rx_bits ~= tx_bits);

end

BER(idx) = errors / (num_symbols * num_tx * 2); % total bits transmitted

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB, BER, 'r*-');

xlabel('SNR (dB)');

ylabel('BER');

title('2x2 MIMO Zero Forcing BER Performance');

grid on;

Result:

Problem 8: Uplink Massive MIMO Simulation with Zero Forcing Detection in 5G NR
Simulate a massive MIMO uplink system in 5G NR with multiple users and antennas. Apply Zero Forcing detection and evaluate BER performance.

- Model an uplink scenario with many users and receive antennas (e.g., 64 antennas).

- Implement Zero Forcing (ZF) linear detection.

- Evaluate system performance (BER vs. SNR).

Program:

clc;

clear;

% Reduced parameters for quick simulation

num_users = 2;            % Number of user equipments (UEs)

num_bs_antennas = 4;      % Number of base station antennas

num_bits_per_user = 100;  % Reduced bits for faster run

SNR_dB = 0:5:30;

BER = zeros(1, length(SNR_dB));

% Generate random bits for each user

bits = randi([0 1], num_users, num_bits_per_user);

% QPSK Modulation

symbols = zeros(num_users, num_bits_per_user / 2);

for user = 1:num_users

tmp = zeros(num_bits_per_user / 2, 1);

idx = 1;

for i = 1:2:num_bits_per_user

b1 = bits(user, i);

b2 = bits(user, i+1);

re = 2*b1 - 1;

im = 2*b2 - 1;

tmp(idx) = (re + 1i*im)/sqrt(2);

idx = idx + 1;

end

symbols(user, :) = tmp.';

end

for idx = 1:length(SNR_dB)

snr = SNR_dB(idx);

errors = 0;

total_bits = 0;

for sym_idx = 1:size(symbols, 2)

x = symbols(:, sym_idx);

% Random Channel Matrix (Massive MIMO channel)

H = (randn(num_bs_antennas, num_users) + 1i*randn(num_bs_antennas, num_users)) / sqrt(2);

% Transmit Signal

y = H * x;

% Calculate signal power

signal_power = mean(abs(y).^2);

% Calculate noise power for given SNR

snr_linear = 10^(snr/10);

noise_power = signal_power / snr_linear;

% Generate noise with appropriate scaling

noise = sqrt(noise_power/2) * (randn(size(y)) + 1i*randn(size(y)));

% Received signal + noise

y_noisy = y + noise;

% Zero Forcing Detection

H_zf = pinv(H);

x_hat = H_zf * y_noisy;

% Demodulate each user

for user = 1:num_users

re = real(x_hat(user));

im = imag(x_hat(user));

rx_bits = [re > 0; im > 0];

tx_bits = [real(x(user)) > 0; imag(x(user)) > 0];

errors = errors + sum(rx_bits ~= tx_bits);

total_bits = total_bits + 2;

end

end

BER(idx) = errors / total_bits;

disp(['SNR = ' num2str(snr) ' dB, BER = ' num2str(BER(idx))]);

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB, BER, 'bs-', 'LineWidth', 1.5);

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('Massive MIMO Uplink with Zero Forcing Detection');

grid on;

Result:

(On licenced tool: num_users =8;            % Number of user equipments (UEs)

num_bs_antennas = 64;      % Number of base station antennas

num_bits_per_user = 1000;  % Reduced bits for faster run)

NR = 0 dB, BER = 0.08

SNR = 5 dB, BER = 0.03

SNR = 10 dB, BER = 0

SNR = 15 dB, BER = 0

SNR = 20 dB, BER = 0

SNR = 25 dB, BER = 0

SNR = 30 dB, BER = 0

Problem 9: Beamforming Radiation Pattern Simulation for a 16-Element Uniform Linear Array (ULA)

Simulate and plot the beamforming radiation pattern of a 16-element ULA used in 5G NR Massive MIMO systems. Steer the beam toward a specified angle (e.g., 30°).

- Create a 16-element ULA antenna array model.

- Implement beam steering to the desired direction.

- Plot the radiation pattern (array factor) in dB.

Program:

clc;

clear;

% Parameters

N = 16;                % Number of antennas in ULA

theta_deg = 30;       % Desired steering angle (degrees)

lambda = 1;           % Wavelength

d = lambda/2;         % Antenna spacing

theta_rad = theta_deg * pi / 180;

% Steering Vector for ULA

n = 0:N-1;

steering_vector = exp(1j*2*pi*d/lambda*n*sin(theta_rad)).';

% Signal transmitted from base station with beamforming

signal = 1;  % Example baseband symbol

tx_signal = steering_vector * signal;

% Assume receiver is in the same direction

rx_signal = steering_vector' * tx_signal;

disp(['Beamformed Received Signal: ', num2str(abs(rx_signal))]);

% Plot antenna array response

theta_scan = -90:1:90;

response = zeros(size(theta_scan));

for idx = 1:length(theta_scan)

t = theta_scan(idx)*pi/180;

sv = exp(1j*2*pi*d/lambda*n*sin(t)).';

response(idx) = abs(sv' * steering_vector);

end

figure;

plot(theta_scan, response);

xlabel('Angle (degrees)');

ylabel('Array Response (Magnitude)');

title('Beamforming Response of ULA');

grid on;

Result:

Input N=16; Output = Beamformed Received Signal: 16

Problem 10: Path Loss Simulation for mmWave Transmission in 5G NR

Simulate the effect of path loss on mmWave signal transmission in 5G NR over varying distances. Plot received power versus distance.

- Model mmWave path loss using standard path loss equations.

- Simulate varying distances and analyze received power.

- Evaluate the impact of path loss on communication range.

Program:

clc;

clear;

% Parameters

freq = 28e9; % 28 GHz mmWave frequency

c = 3e8;     % Speed of light

lambda = c / freq;

Pt_dBm = 30; % Transmit power in dBm

Gt_dB = 15;  % Transmit antenna gain

Gr_dB = 15;  % Receive antenna gain

distances = 1:1:200; % in meters

% Free Space Path Loss Model

FSPL_dB = @(d) 20*log10(d) + 20*log10(freq) - 147.55;

% NLOS additional loss (simple model: 20 dB penalty)

NLOS_penalty_dB = 20;

Pr_LOS_dBm = Pt_dBm + Gt_dB + Gr_dB - FSPL_dB(distances);

Pr_NLOS_dBm = Pr_LOS_dBm - NLOS_penalty_dB;

% Plot

figure;

plot(distances, Pr_LOS_dBm, 'b-', 'LineWidth', 2); hold on;

plot(distances, Pr_NLOS_dBm, 'r--', 'LineWidth', 2);

xlabel('Distance (m)');

ylabel('Received Power (dBm)');

title('mmWave Propagation: LOS vs. NLOS');

legend('LOS', 'NLOS');

grid on;

Result:

Problem 11: Pilot-Based Channel Estimation in 5G NR

Implement simple pilot-based channel estimation for 5G NR. Evaluate the accuracy of channel estimation in the presence of AWGN.

- Transmit pilot symbols in the channel.

- Perform Least Squares (LS) or Minimum Mean Square Error (MMSE) estimation.

- Compare estimated versus actual channel and evaluate BER.

Program:

clc;

clear;

% Parameters

num_subcarriers = 64;

num_pilot_subcarriers = 8;

pilot_indices = 1:8:num_subcarriers;

SNR_dB = 20;

% Generate random data and pilot symbols (QPSK)

data_bits = randi([0 1], num_subcarriers*2, 1);

data_symbols = [];

for i = 1:2:length(data_bits)

re = 2*data_bits(i) - 1;

im = 2*data_bits(i+1) - 1;

data_symbols = [data_symbols; (re + 1i*im)/sqrt(2)];

end

% Assign pilots

pilot_symbol = 1 + 1i; % fixed known pilot

tx_symbols = data_symbols;

tx_symbols(pilot_indices) = pilot_symbol;

% Channel (Random flat fading per subcarrier)

H_true = (randn(num_subcarriers,1) + 1i*randn(num_subcarriers,1)) / sqrt(2);

% Transmit through channel + noise

rx_symbols = H_true .* tx_symbols;

noise = (randn(num_subcarriers,1) + 1i*randn(num_subcarriers,1)) * 10^(-SNR_dB/20);

rx_symbols_noisy = rx_symbols + noise;

% Channel estimation using pilots

H_est = zeros(num_subcarriers,1);

H_est(pilot_indices) = rx_symbols_noisy(pilot_indices) / pilot_symbol;

% Simple interpolation for non-pilot subcarriers (zero-order hold)

for idx = 1:length(pilot_indices)-1

start_idx = pilot_indices(idx);

end_idx = pilot_indices(idx+1);

H_est(start_idx:end_idx) = H_est(start_idx);

end

H_est(end_idx:end) = H_est(end_idx);

% Equalization

eq_symbols = rx_symbols_noisy ./ H_est;

% Demodulation

rx_bits = [];

for k = 1:length(eq_symbols)

re = real(eq_symbols(k));

im = imag(eq_symbols(k));

rx_bits = [rx_bits; re > 0; im > 0];

end

% BER Calculation

errors = sum(data_bits ~= rx_bits);

ber = errors / length(data_bits);

disp(['Estimated BER after Channel Estimation: ', num2str(ber)]);

Result: Estimated BER after Channel Estimation: 0.48438

Problem 12: Device-to-Device (D2D) Communication in 5G NR

Simulate direct Device-to-Device (D2D) communication in 5G NR and compare BER performance with traditional cellular communication using relays.

- Model direct D2D communication and cellular relay-based communication.

- Analyze BER versus SNR for both approaches.

- Highlight the benefits of D2D communication in terms of spectral efficiency and latency.

Program:

clc;

clear;

% Parameters

num_bits = 10000;

SNR_dB = 0:2:20;

ber_d2d = zeros(size(SNR_dB));

ber_relay = zeros(size(SNR_dB));

for i = 1:length(SNR_dB)

% Generate random bits

bits = randi([0 1], num_bits, 1);

% BPSK Modulation

tx_symbols = 2*bits - 1;

% D2D Direct Link

noise = randn(size(tx_symbols)) * 10^(-SNR_dB(i)/20);

rx_symbols_d2d = tx_symbols + noise;

rx_bits_d2d = rx_symbols_d2d > 0;

% Relay Communication (Two hops, each with SNR loss)

noise1 = randn(size(tx_symbols)) * 10^(-(SNR_dB(i)-3)/20);

relay_signal = tx_symbols + noise1;

noise2 = randn(size(tx_symbols)) * 10^(-(SNR_dB(i)-3)/20);

rx_symbols_relay = relay_signal + noise2;

rx_bits_relay = rx_symbols_relay > 0;

% BER Calculation

ber_d2d(i) = sum(bits ~= rx_bits_d2d) / num_bits;

ber_relay(i) = sum(bits ~= rx_bits_relay) / num_bits;

end

% Plot

figure;

semilogy(SNR_dB, ber_d2d, 'b-o', 'LineWidth', 2); hold on;

semilogy(SNR_dB, ber_relay, 'r-s', 'LineWidth', 2);

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('D2D vs. Relay Communication in 5G');

legend('D2D Direct Link', 'Relay Communication');

grid on;

Result: