Problem 1: Simulation and Analysis of 16-QAM Modulation in 5G NR

Implement and analyze a 16-QAM modulation scheme for 5G NR systems. Evaluate the Bit Error Rate (BER) performance over an AWGN channel and plot BER versus SNR.

- Develop a MATLAB/Octave simulation of 16-QAM modulation and demodulation.

- Transmit data through an AWGN channel.

- Analyze BER performance as a function of SNR.

Program:

clc;

clear;

close all;

% Parameters

M = 64; % Modulation order (4, 16, 64)

k = log2(M); % Bits per symbol

num_bits = 1000; % Total number of bits

% Generate random bits

bits = randi([0 1], num_bits, 1);

% Ensure length of bits is a multiple of k

num_symbols = floor(length(bits) / k);

bits = bits(1:num_symbols * k);

% Reshape bits into k-bit groups (each row = 1 symbol)

bit_groups = reshape(bits, k, num_symbols).';

% Manual binary to decimal conversion (replace bi2de)

powers_of_two = 2.^(k-1:-1:0);

symbol_indices = bit_groups * powers_of_two.';  % decimal symbol indices

% Generate QAM constellation points (square QAM only)

m_sqrt = sqrt(M);

if mod(m_sqrt, 1) ~= 0

error('M must be a perfect square for square QAM');

end

% Create I and Q points

I = -(m_sqrt - 1):2:(m_sqrt - 1);

Q = -(m_sqrt - 1):2:(m_sqrt - 1);

% Create meshgrid and flatten it

[II, QQ] = meshgrid(I, Q);

constellation = II(:) + 1j * QQ(:);

% Normalize constellation power to 1

constellation = constellation / sqrt(mean(abs(constellation).^2));

% Modulate: map indices to constellation points

tx_symbols = constellation(symbol_indices + 1);

% Plot transmitted symbols (Constellation Diagram)

figure;

scatter(real(tx_symbols), imag(tx_symbols), 'filled');

xlabel('In-Phase');

ylabel('Quadrature');

title([num2str(M) '-QAM Constellation (Transmitted Symbols)']);

grid on;

% Add AWGN Noise (Optional)

snr_db = 20; % SNR in dB

snr_linear = 10^(snr_db/10);

noise_variance = 1 / (2 * snr_linear);

noise = sqrt(noise_variance) * (randn(size(tx_symbols)) + 1j * randn(size(tx_symbols)));

rx_symbols = tx_symbols + noise;

% Demodulate: Find nearest constellation point

distances = abs(rx_symbols - constellation.');

[~, demod_indices] = min(distances, [], 2);

% Manual decimal to binary conversion (replace de2bi)

demod_bits = zeros(num_symbols, k);

for i = 1:k

demod_bits(:, i) = bitget(demod_indices - 1, k - i + 1);

end

% Reshape to column vector

demod_bits = reshape(demod_bits.', [], 1);

% Compute BER (Bit Error Rate)

num_errors = sum(bits ~= demod_bits);

ber = num_errors / length(bits);

% Display results

disp(['Number of bit errors: ' num2str(num_errors)]);

disp(['Bit Error Rate (BER): ' num2str(ber)]);

% Plot received symbols (With noise)

figure;

scatter(real(rx_symbols), imag(rx_symbols), 'filled');

xlabel('In-Phase');

ylabel('Quadrature');

title([num2str(M) '-QAM Constellation (Received Symbols with Noise, SNR = ' num2str(snr_db) ' dB)']);

grid on;

Results:

M=4

Number of bit errors: 0

Bit Error Rate (BER): 0

M=16

Number of bit errors: 0

Bit Error Rate (BER): 0

M=64

Number of bit errors: 11

Bit Error Rate (BER): 0.011044

Problem 2: Adaptive Modulation and Coding (AMC) in 5G NR

Simulate an Adaptive Modulation and Coding (AMC) mechanism in 5G NR that dynamically selects modulation schemes (BPSK, QPSK, 16-QAM) based on varying SNR conditions.

- Implement SNR-based modulation adaptation.

- Evaluate BER and throughput performance for each modulation type.

- Demonstrate improved system efficiency through adaptive selection.

Program:

clc;

clear;

% Define SNR range

snr_range = 0:2:20;

% Thresholds for adaptive modulation

snr_threshold_qpsk = 5;

snr_threshold_16qam = 15;

% Initialize result storage

bits_per_symbol_all = zeros(size(snr_range));

% Loop over SNR range

for idx = 1:length(snr_range)

snr = snr_range(idx);

% Adaptive modulation selection

if snr < snr_threshold_qpsk

M = 2; % BPSK

scheme = 'BPSK';

elseif snr < snr_threshold_16qam

M = 4; % QPSK

scheme = 'QPSK';

else

M = 16; % 16-QAM

scheme = '16-QAM';

end

% Save results

bits_per_symbol = log2(M);

bits_per_symbol_all(idx) = bits_per_symbol;

% Display decision

disp(['SNR: ', num2str(snr), ' dB - Modulation: ', scheme, ' - Bits per symbol: ', num2str(bits_per_symbol)]);

end

% Plot bits per symbol vs SNR

figure;

plot(snr_range, bits_per_symbol_all, '-ob', 'LineWidth', 2, 'MarkerSize', 6);

grid on;

xlabel('SNR (dB)');

ylabel('Bits per Symbol');

title('Adaptive Modulation: Bits per Symbol vs SNR');

Result:

SNR: 0 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 2 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 4 dB - Modulation: BPSK - Bits per symbol: 1

SNR: 6 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 8 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 10 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 12 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 14 dB - Modulation: QPSK - Bits per symbol: 2

SNR: 16 dB - Modulation: 16-QAM - Bits per symbol: 4

SNR: 18 dB - Modulation: 16-QAM - Bits per symbol: 4

SNR: 20 dB - Modulation: 16-QAM - Bits per symbol: 4

Problem 3: 5G NR Channel Coding using Parity-Check Codes

Implement basic channel coding in 5G NR using parity-check codes. Demonstrate error detection capability over an AWGN channel.

- Encode data using parity-check coding.

- Simulate transmission over an AWGN channel.

- Implement error detection and evaluate BER improvement.

Program:

clc;

clear;

% Parameters

M = 4; % QPSK

num_bits = 10000;

SNR_dB = 0:5:30;

BER = zeros(size(SNR_dB));

% Generate random bits

bits = randi([0 1], num_bits, 1);

% Manual QPSK Modulation

symbols = [];

for i = 1:2:num_bits

b1 = bits(i);

b2 = bits(i+1);

re = 2*b1 - 1;

im = 2*b2 - 1;

symbols = [symbols; (re + 1i*im)/sqrt(2)];

end

for idx = 1:length(SNR_dB)

snr = SNR_dB(idx);

% Add AWGN

noise = (randn(size(symbols)) + 1i*randn(size(symbols))) * 10^(-snr/20);

rx = symbols + noise;

% Demodulation

rx_bits = [];

for k = 1:length(rx)

re = real(rx(k));

im = imag(rx(k));

rx_bits = [rx_bits; re > 0; im > 0];

end

% BER

errors = sum(bits ~= rx_bits);

BER(idx) = errors / num_bits;

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB, BER, 'bo-');

xlabel('SNR (dB)');

ylabel('BER');

title('QPSK over AWGN');

grid on;

Result: