clc;

clear;

% --------------------------

% Parameters

% --------------------------

N = 64;              % Number of subcarriers

CP_len = 16;         % Cyclic prefix length

num_symbols = 100;   % Number of OFDM symbols per SNR

M = 4;               % QPSK (2 bits per symbol)

SNR_dB_range = 0:2:20; % SNR range in dB for BER plot

num_bits_per_symbol = log2(M);

num_total_bits = num_symbols * N * num_bits_per_symbol;

BER = zeros(length(SNR_dB_range), 1); % Initialize BER array

% --------------------------

% Start BER vs. SNR Simulation

% --------------------------

for idx = 1:length(SNR_dB_range)

snr_db = SNR_dB_range(idx);

% --------------------------------------

% Transmitter: Generate random data bits

% --------------------------------------

bits = randi([0 1], num_total_bits, 1);

% --------------------------------------

% QPSK Modulation (manual)

% --------------------------------------

symbols = zeros(length(bits)/2, 1);

for i = 1:2:length(bits)

b1 = bits(i);

b2 = bits(i+1);

% QPSK mapping: Gray coding

re = 2 * b1 - 1;

im = 2 * b2 - 1;

symbols((i+1)/2) = (re + 1i * im) / sqrt(2);

end

% --------------------------------------

% OFDM Modulation

% --------------------------------------

ofdm_symbols = reshape(symbols, N, num_symbols);      % Reshape to N subcarriers

tx_signal = ifft(ofdm_symbols);                       % IFFT for OFDM modulation

tx_cp = [tx_signal(end - CP_len + 1:end, :); tx_signal]; % Add cyclic prefix

tx_serial = tx_cp(:);                                 % Serialize

% --------------------------------------

% Add AWGN Noise (manual)

% --------------------------------------

signal_power = mean(abs(tx_serial).^2);

snr_linear = 10^(snr_db / 10);

noise_power = signal_power / snr_linear;

noise = sqrt(noise_power / 2) * (randn(size(tx_serial)) + 1i * randn(size(tx_serial)));

rx_serial = tx_serial + noise;

% --------------------------------------

% OFDM Demodulation

% --------------------------------------

rx_cp = reshape(rx_serial, N + CP_len, num_symbols);

rx_signal = rx_cp(CP_len + 1:end, :);

rx_ofdm_symbols = fft(rx_signal);

rx_symbols = rx_ofdm_symbols(:);

% --------------------------------------

% QPSK Demodulation (manual)

% --------------------------------------

rx_bits = zeros(length(bits), 1);

for k = 1:length(rx_symbols)

re = real(rx_symbols(k));

im = imag(rx_symbols(k));

rx_bits(2*k - 1) = re > 0; % First bit (real part)

rx_bits(2*k)     = im > 0; % Second bit (imag part)

end

% --------------------------------------

% BER Calculation

% --------------------------------------

num_errors = sum(bits ~= rx_bits);

BER(idx) = num_errors / length(bits);

disp(['SNR = ' num2str(snr_db) ' dB, BER = ' num2str(BER(idx))]);

end

% --------------------------

% Plot BER vs. SNR

% --------------------------

figure;

semilogy(SNR_dB_range, BER, '-o', 'LineWidth', 2);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('BER vs. SNR for QPSK-OFDM (No External Functions)');

legend('OFDM-QPSK');

Result:

Problem 5: OFDM Transmission over Multipath Fading Channels with Cyclic Prefix in 5G NR

Simulate a 5G NR OFDM system with Cyclic Prefix (CP) and evaluate its performance over multipath fading channels.

- Implement Cyclic Prefix addition and removal.

- Model a multipath fading channel.

- Analyze BER performance under various SNRs and multipath conditions.

Program:

clc;

clear;

% Parameters

num_subcarriers = 64;

num_symbols = 50;  % Reduced for faster simulation

cp_len = 16;

SNR_dB_range = 0:5:30;

% Multipath Channel

h = [0.8; 0.5*exp(1j*pi/4); 0.3*exp(1j*pi/3)];

% BER Initialization

BER = zeros(1, length(SNR_dB_range));

% SNR loop

for snr_idx = 1:length(SNR_dB_range)

SNR_dB = SNR_dB_range(snr_idx);

errors = 0;

total_bits = 0;

% Generate bits

num_bits = num_subcarriers * num_symbols * 2; % 2 bits per QPSK symbol

bits = randi([0 1], num_bits, 1);

% QPSK modulation

symbols = zeros(num_subcarriers, num_symbols);

for sym_idx = 1:num_symbols

temp_symbols = zeros(num_subcarriers, 1);

for sc = 1:num_subcarriers

idx = (sym_idx - 1) * num_subcarriers * 2 + (sc - 1) * 2 + 1;

b1 = bits(idx);

b2 = bits(idx + 1);

temp_symbols(sc) = (2*b1 - 1 + 1j*(2*b2 - 1)) / sqrt(2);

end

symbols(:, sym_idx) = temp_symbols;

end

% OFDM Modulation + CP insertion

tx_signal = [];

for i = 1:num_symbols

ifft_data = ifft(symbols(:, i));

ofdm_symbol_cp = [ifft_data(end - cp_len + 1:end); ifft_data]; % Add cyclic prefix

tx_signal = [tx_signal; ofdm_symbol_cp];

end

% Channel convolution

channel_output = conv(tx_signal, h);

channel_output = channel_output(1:length(tx_signal));  % Trim extra samples

% Add AWGN

signal_power = mean(abs(channel_output).^2);

snr_linear = 10^(SNR_dB / 10);

noise_power = signal_power / snr_linear;

noise = sqrt(noise_power / 2) * (randn(size(channel_output)) + 1j * randn(size(channel_output)));

rx_signal = channel_output + noise;

% Receiver: OFDM Demod + CP removal + Equalization

rx_symbols = zeros(num_subcarriers, num_symbols);

index = 0;

for i = 1:num_symbols

start_idx = index + cp_len + 1;

end_idx = index + cp_len + num_subcarriers;

if end_idx > length(rx_signal)

disp('Reached end of rx signal');

break;

end

r = rx_signal(start_idx:end_idx);

index = index + num_subcarriers + cp_len;

rx_ofdm = fft(r);

% Channel frequency response

H = fft([h; zeros(num_subcarriers - length(h), 1)]);

% Zero Forcing Equalizer

eq_ofdm = rx_ofdm ./ H;

rx_symbols(:, i) = eq_ofdm;

end

% Demodulation

rx_bits = zeros(num_bits, 1);

symbol_counter = 0;

for i = 1:num_symbols

for k = 1:num_subcarriers

symbol_counter = symbol_counter + 1;

rx_sym = rx_symbols(k, i);

rx_bits(2*symbol_counter - 1) = real(rx_sym) > 0;

rx_bits(2*symbol_counter) = imag(rx_sym) > 0;

end

end

% BER calculation

bit_errors = sum(bits ~= rx_bits);

BER(snr_idx) = bit_errors / num_bits;

disp(['SNR = ', num2str(SNR_dB), ' dB, BER = ', num2str(BER(snr_idx))]);

end

% Plot BER vs SNR

figure;

semilogy(SNR_dB_range, BER, 'ro-', 'LineWidth', 2);

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('OFDM BER vs SNR over Multipath Channel');

grid on;

legend('QPSK-OFDM');

Result:

SNR = 0 dB, BER = 0.20703

SNR = 5 dB, BER = 0.090781

SNR = 10 dB, BER = 0.026562

SNR = 15 dB, BER = 0.0034375

SNR = 20 dB, BER = 0

SNR = 25 dB, BER = 0

SNR = 30 dB, BER = 0

Problem 6: Exploring 5G NR Numerology (Subcarrier Spacing and CP Lengths)

Simulate different 5G NR numerologies by varying subcarrier spacing (SCS) and cyclic prefix lengths. Evaluate their impact on system performance.

- Implement multiple numerology configurations (e.g., 15 kHz, 30 kHz SCS).

- Analyze their effects on bandwidth, latency, and BER.

- Compare the trade-offs between different numerologies.

Program:

clc;

clear;

% Base parameters

f_ref = 15e3; % 15 kHz base subcarrier spacing

numerologies = 0:4; % µ = 0 to 4

fs = 30.72e6; % Sampling rate (for simplicity)

% Arrays to store results

SCS = zeros(size(numerologies));

CP_durations = zeros(size(numerologies));

for i = 1:length(numerologies)

mu = numerologies(i);

SCS(i) = f_ref * 2^mu; % Subcarrier spacing

Nfft = fs / SCS(i);

Ncp = Nfft * 0.07; % Assume CP is 7% of symbol length

CP_durations(i) = Ncp / fs * 1e6; % in microseconds

end

% Plot

figure;

subplot(2,1,1);

stem(numerologies, SCS/1e3, 'LineWidth', 2);

xlabel('Numerology Index (µ)');

ylabel('Subcarrier Spacing (kHz)');

title('5G NR Numerology: Subcarrier Spacing');

grid on;

subplot(2,1,2);

stem(numerologies, CP_durations, 'r', 'LineWidth', 2);

xlabel('Numerology Index (µ)');

ylabel('CP Duration (\mus)');

title('5G NR Numerology: CP Duration');

grid on;

Result: